Development of an Object-Oriented Finite Element Analysis Software

by

Alexandre LANDREAU

Strathclyde University

Mechanical Engineering Department

26/04/2002

Supervisor : Tugrul COMLEKCI

Acknowledgments

Dr T Comlekci contributes to a large part of the success of the project. His implication by his explanation helps me a lot. I gave him all my regards for his actions.

Abstract

Strathclyde University is planning to develop a computing system, which is able to solve complex structural problems using stochastic finite element analysis.

Mechanics problems could have some random properties or characteristics and that can affect the structures (stress distribution, vibrations). Stochastic Object-Oriented Finite Element Analysis software (SOFEA) will help analysts, with an interest in computer-aided engineering of such problems.

Stochastic finite element method requires large computer resources, and data processing can be improved using parallel computing (e.g. Parallel Virtual Machine), a high performance and low cost method.

The concerns of this report are about the mainframe work of the global project, specifically a finite element solver has to be designed, using an object-oriented language. Stochastic methods will be the subject of future projects in the department of mechanical engineering.

Technology evolves and the ancient languages of programming start to become inefficient regarding the software development and the code compilation. A part of the report is about the object-oriented method and its advantages.

To fulfil this project, an object-oriented language was chosen. With its help, the code solves ‘2D’ problems with a few finite element types such as spar, beam and plane elements. This program doesn’t include graphical interface for the pre and post processor.

This report will demonstrate the power of the object-oriented languages such as C++ and show the results of linear elastic case studies.

Contents

1	Introduction	6

2	Background of SOFEA	7

2.1	Finite Element Analysis : a brief introduction	7

2.1.1	Structural problem solving.	8

2.2	Equation of Equilibrium	10

2.3	Stochastic Finite Element Analysis	13

2.4	Objectives	14

2.5	Specifications, constraints	14

2.6	Work organisation - plan	15

2.6.1	Planning	15

2.6.2	Literature survey	16

2.6.3	Programming functions	16

3	The Finite Element Programming	17

3.1	Advantages of Object orientation	17

3.1.1	Introduction to Object–oriented program development	17

3.1.2	Theory of the object and pointer	19

3.1.3	The portability of the software	22

3.2	The program flow chart (See Appendix 6 : The Code)	23

3.2.1	Pre and post processor	23

3.2.2	Creation of Classes	24

3.2.3	The Master matrix assembly	33

3.2.4	The matrix solving	36

3.2.5	Matrix solvers	37

3.2.6	Result output	41

4	Results	42

4.1	Ansys tools	42

4.2	Case studies	42

4.3	Results	44

4.4	Analysis	45

5	Code improvements and future improvements	45

5.1	Debugging	46

5.2	Errors due to the final users	46

5.3	Source code improvement	47

5.4	Classes implementation	47

6	Conclusion	48

7	Appendices	50

7.1	Appendix 1: algorithm	50

7.2	Appendix 2 : the data file	54

7.3	Appendix 3: Class example	56

7.4	Appendix 4 : FEView	57

7.5	Appendix 5 : new FEA frontier	58

7.6	Appendix 6 : The Code	61

7.7	Appendix 7 : Isoparametric	110

8	References	114

# Introduction

Nowadays, companies or universities are interested in decreasing their product development and research costs by reducing the amount of experiments for solving problems. Computers can help a research and development department if engineers use specific analysis applications.

A new research area in stochastic finite element methods is being launched at the University of Strathclyde. The global project consists of designing a computing system capable of solving complex structural problems with the finite element method. Many data and parameters could vary along the structure, or are difficult to measure (such as load and material properties). The stochastic method searches the best results we can obtain.

The first efforts of the project are focussed on a main framework for the FE program. During this year, the work done has been oriented on solving 2D problems using a few types of elements. A FE code solving linear elastic problems using basic elements such as a spar, a beam and plane elements is to be delivered at the end of the year.

New technologies relating to languages of programming has been involved i.e. the choice of C++ as a standard language was an important step because it uses objects and simplifies the work of developers. This report describes the advantage of using this technology and explains how to use it with finite element analysis.

The programming task has been detailed in this paper, and the most interesting tasks described in it. The code can be separated in 5 parts. To validate the program, some case studies have been run and the most significant reported here.

Finally, several ideas have been found along the project to improve it. They could be the starting point of the next years project.

# Background of SOFEA

University of Strathclyde is planning a new method for solving structural problems. This method employs finite element analysis and stochastic methods and will give to the users more information for decision-making. By creating many scenarios, the engineers will assess the risks and be able to make decision (e.g. avoid failure due to random material properties)

The global project is to design a program working on a multiprocessor computer and using an UNIX operating system. The program may be dedicated specially for this machine to utilize the computer resources with great efficiency. This year, the project aim was to start the project by designing a Finite Element analysis using object-oriented software development technology.

Finite Element Analysis and stochastic method were introduced to give a general aspect of the problem. Then the manner of the project was driven has been explained, i.e. work organisation.

## Finite Element Analysis : a brief introduction

This method was discovered at the mid century. Unfortunately, it requires a lot of calculations and at this period, computers did not exist. Performing to these calculations by hand takes too much time and involves too many risks of error. The idea was given up till the discovery of the computer

To solve a structural problem, we can choose a suitable method adapted for the problem. However finding an exact solution of a problem is to define the exact equation of the solution. Due to the complexity of certain structural problems, finding an exact solution remains impossible for scientists or engineers. The finite element method is an approximation method, which simplifies the problem to give to the solution enough accuracy for the needs of its users.

### Structural problem solving.

The solution of solid mechanics problems has to satisfy governing equations and the boundary conditions. The governing equations describe the system.

The field conditions such as variables, displacements and stresses must satisfy the governing equation. The field variables satisfy the equations for the equilibrium and the continuity.

These equations are usually too complex to solve for many engineering problems. An exact analytical solutions demand time to solve due to the complexity of the problem. Then engineers prefer to use approximate solutions. The accuracy is enough for engineering problems. To understand what finite element method is, a summary of 3 methods has been done :

- Functional approximation.

- Finite difference method.

- Finite element method.

The functional approximation is the same as some well-known classical methods : Rayleigh-Ritz, Timoshenko… For Rayleigh-Ritz method, the engineer describes with errors the displacement by a function (e.g. : y=a1sinπxL+a2sinπxL). Then he calculates the strain energy U and the potential energy H. The total potential energy is the difference of the two last calculated energy. He finds an expression function of unknown parameters. The partial derivative of the total potential energy by these unknowns is equal to zero (∂Π∂ai). The approximate result is found. The accuracy of the results depends on the result and of the function, which model the displacement and the type of the method (e.g. Timoshenko more accurate for buckling of thin-walled structures).

The finite difference method was a method used to solve difficult problems of structural mechanics. The body is ‘discretized’ by a mesh of nodal points.

The field variable is represented by discrete values of the variable at the ‘node’. The displacements for most of the cases are considered as unknown. The resulting equations are then solved for the nodal values of this variable. The solving process is made easier by the symmetry of the boundary conditions and the loading.

The accuracy of the result depends on of the number of the nodes. Moreover the results obtained by this method are accurate enough to be used in industry.

This procedure is quite hard to use in computerisation because the differential equations which become quite involved to derive for irregular shapes. Also if the material is not isotropic, the governing differential equation will become complex.

Even if the method is not suitable for computerisation, the concept of ‘discretization’ is remarkable and is used in a newer method : finite element method.

Finite element method combines the feature of two approximate methods of analysis, previously introduced. The method is fully suitable for computer programming and offers solutions to a wide range of problems.

The finite element method consists of dividing a body in small elements called ‘finite element’. The properties of each element are formulated and combined to obtain the solution of the global body.

For the displacement formulation, simple functions called ‘ shapes function’ are chosen to approximate the displacement in the element. This displacement concerns only the node. Rayleigh-Ritz method is used at the element level. The strains and stresses in the element are also expressed in terms of nodal displacements. Finally the use of the principle of virtual displacement or minimum potential energy is used to derive the equation of equilibrium for the element and the nodal displacements will be the unknowns in the equations.

The equilibrium of the entire structure is determined by equations, which are the result of the combination of the equation of equilibrium of each element and the continuity of the displacement for each node.

Several kinds of problem can be solved with the help of the number of type of element (bar element, plane element, other 3D elements).

## Equation of Equilibrium

Let’s consider a small volume around M, which in equilibrium of body forces Fx, Fy and Fz. On each side there are a normal stress and 2 tangential stresses.

Figure 1 : Equilibrium in the x direction

On the axis x, if the normal stress is called xx, the force applied is -xxdydz. The sum of the forces in all the direction is null. If we express it on x axis, this is:

∂σxx∂x+∂σyx∂y+∂σzx∂z+Fx=0

With a similar reasoning, the sum of forces applied on x and y forces are null. From these formulas, a generic one can state :

σij,i+Fj=0

This equation is valid in all the body and is considered as a basis for continuity.

Strains :

The definition of the strain is function of the displacement on k direction :

εij=12∂ui∂xj+∂uj∂xi and εij=εji

For example, a linear spring has a linear deformation : the strain is ε=ul.

In linear mechanics, one must check that all the strains or distortion are small to accept the calculation’s assumption.

Hooke made some research on the relationship between stresses and strain. He determines a matrix called Hooke’s matrix (H). The values of this matrix depends on the nature of the body e.g. a plate or a bar.

In our project, the solving process is restricted to linear static calculation i.e. the solving of classic problem F=Kq where F is the forces vector, q displacement vector (Values are small) and K is the constant stiffness matrix.

The definition of the stiffness matrix is : K=BtHB dv, where B is called the strain-stress matrix and is equal to B=ddsN (N is the shape function matrix) and H is the Hooke matrix.

The application of the theory to the computing is slightly different e.g. there is no formulas called integrate in any language. The method has to be different.

A calculation for linear elastic analysis has 6 six different steps:

- Translation of user’s data in Finite Elements.

- Calculation of the local stiffness matrix.

- Assembly of the global matrix.

- Boundary conditions treatment.

- Displacement determination.

- Stress Calculation

## Stochastic Finite Element Analysis

Stochastic is described in this paragraph to understand what are the concerns of the global project. The current programming does not take into account stochastic methods. To understand the project, a definition of stochastic and some example of method are given.

Dictionary definition :

- RANDOM; specially : involving a random variable (a stochastic problem).

- involving chance or probability : PROBABILISTIC

Generally, a stochastic process is a process involving a random element. Stochastic processes are important in non-equilibrium statistical mechanics. To analyse stochastic processes and their equation, it is necessary to use statistical methods and the theory of probability.

Two well-know methods are quoted : Markov chain and Monte-Carlo method.

The following example is to explain how stochastic method works. The name of Monte-Carlo is from the roulette in Monaco, which is considered as a mechanism to draw numbers randomly. This technique is used scientifically and financially.

Uncertainties affecting the state of a part are quantified with the distribution of probabilities. A large number of scenarios are created during the simulation from samples of the probabilities. The state of the part can be calculated considering each one of the scenarios. With the help of this analysis, a distribution of the probability of the result is obtained (stresses, displacements, …). A large range of results is given and then the user can assess the risks of failure.

## Objectives

The objective of the project is to design a program solving a structural problem with the finite element method. The choice of the language has to be defined but must be an object-oriented language. At the end of the year, a tested program has to be delivered in order to solve structures with simple elements such as bar, beam or plane elements.

A web site will group the information and will detail the project. From this website, we will access to other website as reference or paper available (as a pdf document, etc).

## Specifications, constraints

In the future, other researches and students may participate in the global project, and may not have real programming skills as professional software developers. If this program could be understandood by all users, it has to be simple. Using comprehensible names for variables, functions,

Instead of losing time to understand what the functions do, the choice of a descriptive name can avoid the “decrypting task”. To name functions, variables helps a lot the reading of the program.

Also the calculation of the global matrix and its assembly are two different methods of the same class, it allows the program to be more understandable during its reading by the programmer.

The code of the program should be an easy source code and understandable; the use of object contributes to this.

The program was ruled with more constraints to obtain a better result. There was willingness to make the program more efficient as possible, and to achieve it, stern efforts were done regarding the low running time, automatic procedures, the optimisation of the memory space and the integration of other classes in the code. Regarding to the global project, operation can be repeated many thousand times, which means inefficient functions, can become disastrous regarding to the total runtime.

## Work organisation - plan

Some efforts were done on the organisation of the project, which helped to complete the project successfully. First of all, the project was planned in order to define objectives to accomplish. To find out about related technologies, some research was done on the Internet but also on special database.

### Planning

The project was shared onto two parts :

For the first semester, the objectives were to understand the use of object-oriented language and the implementation of 2 basic elements. Once this first task was completed, an application was thought involving an element i.e. a link element with 2 degree of freedom per node. The program main frame was formed. Then the integration of a second element (beam element) was done.

For the second semester, some errors during the execution of the code were identified with simple case studies and debugged. The first task was to solve these problems. Then the second task was to implement a more complex element i.e. a plane element. And finally a web site has to be designed to explain the project.

### Literature survey

To complete successfully some object-oriented classes or to understand either how the finite element analysis works or objects, some researches was done by making enquiries on internet, library or databases. Books on finite element method were used as references for the program. To complement this information, further research was done on papers, which were found on BIDS database. Also on Internet, many scientists presented their research on a website and many documents on C++ or finite element method was found.

### Programming functions

Before beginning programming on computer, functions were planed on paper using a flow chart. Programming demands accuracy during the phase of programming i.e. each little error transforms the code to a non-readable program or giving errors. It is important to get the right algorithm. For example, if two functions were designed, one on the local matrix calculation and the second on the assembly of the matrix, and if a mistake has been made in the assembly of the matrix and then when the program is running, the developer doesn’t know where the error is and need to use the debugger. The debug process takes a lot of time and it can be avoided by designing a good code at the beginning with the help of an algorithm (See Appendix 1: algorithm).

To avoid any error of interpretation, a weekly report has been done about the work complemented. Often a meeting was organised to discuss about the improvements and the objectives.

# The Finite Element Programming

## Advantages of Object orientation

### Introduction to Object–oriented program development

Since the computer appeared, languages were adapted for the computers’ processor. However the change of languages remains slower due to the difficulty of updating codes to a new language. Nowadays the standard is to program with an object-oriented language such as C++.

Thirty years ago, scientists saw in computers future tools to help them in their research, even if the first compilers were unreliable and the program execution very slow. The instructions to the machine had to be simple and quite short in order to make it works with efficiency. FORTRAN was quickly adopted as the standard for scientists who started to develop the first program of finite elements.

So far, massive improvements were done i.e. a new brand of computer processor go out every 6 months with more and more transistors and its frequency of work doesn’t stop going up. So, codes had been developed to take advantage of these new computers, which in some cases provide, more than a TeraFLOPs.

New technology aspires new technology and nowadays the old compilers are not efficient for the new hardware and that justify the elaboration of new languages.

Currently, the developers tend to do much longer codes. The way of programming is more complex and that’s why techniques were adapted for the new languages. They were developed for a faster program execution but also for a faster and better manner of development i.e. the work of developers becomes easier.

Problems of maintenance, readability and changes tend to be reduced with new languages. The development software offers much more tools as powerful debuggers, graphics interfaces, etc …

The integration of new libraries with the use of objects is possible. Many websites offer mathematics libraries, or graphical libraries (openGL). The work between different developers is easier also, and they began team working on the same program with the use of encapsulation. The library provides a class including variables and functions called methods. Libraries implemented in the program provide to the developers new functions, which will be used to manipulate the objects. Developers have to know what they do but not how they do it.

The developer can use the class by creating another class, which inherits the properties of its ancestor. This method is called inherence. The developers can use methods already done and programs his own or add new variables.

Among the huge list of object-oriented languages, we can quote the most famous ones : Java and C++. Both are standard but in different uses. Java is a language for web-based applications while C++ is a language for a program, which can be under an operating system. In the frame of our project, the choice of C++ is a good means to replace FORTRAN. Scientists and engineers have adopted it easily.

The choice of C++ is quite adapted to the requirement of the project. The object-oriented languages own favourable properties, such as the portability. The program has been developed and tested under Windows but we have to remind that the computer where it will run has a Linux OS. The code remains the same, the compiler changes for a system to another. Indeed, an operating system is an interface between the processor and an application, and the commands to the operating system are different between Linux and Windows.

Furthermore, C++ is very wide spread that it makes the research of developers knowing this language, so easier. On the Internet, many websites give helps, tips, tools, … about the software.

Finally C++ is really better than C for scientists and engineers. To have the same result, C will require more lines and more knowledge. C++ manages better the memory better and the use of structures is easy compared to C. More and more scientists use C++ because it is a high-level software, which reduces errors due to the way of programming.

### Theory of the object and pointer

Pointer and arrays of pointers :

They were created to manipulate indirectly the data. They help the writing for the program.

Pointer :

It is a variable, which points to another variable. Define a pointer means that to know the memory address of a variable. The same pointer can point on different variables one after another and the value of the pointed variable can be read or written.

Example for a character string :

// Definition of a string

char String01[] = ‘S’,’t’,’r’,’a’,’t’,’h’,’C’,’l’,’y’,’d’,’e’,’\n’;

char String02[] = ‘U’,’n’,’i’,’v’,’e’,’r’,’s’,’i’,’t’,’y’,’\n’;

char *Pointer;

// Pointer points on the first character of the String01

Pointer= &String01[0];

The use of pointers allowed to develop more automatic procedures. To carry on the example, a function can be created to work on strings.

WorkOn (&String01[0],4);

WorkOn (&String02[0],4);

.

.

.

WorkOn (char *Read, int Count)

{

// Loop to do “Count” time(s)

for (int inc=0; inc<Count;inc++)

{

// Display the character that Read points on

cout<<”character “<<inc+1<<” : “<<*Read;

// Point on the next character

Read++;

}

}

Listing 1 : example for the use of a pointer

The pointer variable is defined as following :

*Name_of_the_variable

To have an example how to use a pointer, the Listing 1 should be study.

Remark : The incrementation for the pointer depends on the nature of its definition. If the variables is coded on ‘x’ bytes, during the incrementation the processor will calculate a new address ‘New address = old address + x’.

Figure 2 : An array of pointers pointing to stings.  Each row has a different to point on it. It is an array with 2 dimensions because the string is also an array ! (adapted from [])

Array of pointer (see Figure 2)

Basically, it is an array of pointers, which point on variables or objects. An array of pointer is similar as 2D dimension array.

This method allows the automatic storage of objects of the same nature. The array gets the addresses of all new objects created.

For the concerns of SOFEA program, these methods are fit to the creation of new elements or new nodes.

During the initialisation of such arrays, a value of 3000 was set up... This value can be seemed as large but actually is the size of 3000 addresses, which is nothing, comparing to the size of storage of current computers.

### The portability of the software

The global project will consist of driving a multiprocessor computer and make the program running. To develop such interface, the best is to develop the program on a special system designed for professional development. A computer with UNIX environment allows low-level development. The SOFEA program has to run under Unix system.

However, the program has been developed on a windows laptop for practical reason. The program has to work either under windows system or under Unix system after recompilation. We call this skill for codes, the portability. C++ is perfectly portable from one system to another system if the compiler is available.

The choice of C++ as a high level language is very strategic. The project can benefit from its advantages i.e. objects, good integration, pointers... The C++ has become a standard in software development and many compilers for different operating systems.

## The program flow chart (See Appendix 6 : The Code)

### Pre and post processor

In much commercial finite element software, users make the model of a part with a graphics interface. Then they obtain stress or deformed shape plots. This is called the pre and post processor.

However, developing our own pre and post processor takes a long time and need knowledge relating to graphical libraries such as OpenGL. It was not the purpose of the project and concentration was more on the solving method.

To replace the graphical interface, a solution involving a command file was retained. Special functions were stated allowing this text file described with a detailed manner the model, the position of nodes, the elements configurations, the material properties, and the boundary conditions such as forces and displacements. A typical data file is included in appendix and functions are described in it.

This system is OK for basic shapes such as a rectangle but remains complicated as soon as the number of nodes increases.

For basics shapes, the user can write commands in a text file. However problems occurred when the meshing requires a large number of nodes. The risk of errors grows as the number of element become more important. So we use the pre processor of a commercial software (ANSYS) and translate the information for nodes and elements into a format that SOFEA is able to understand (See Appendix 2 : the data file).

### Creation of Classes

The use of C++ brings advantages to the developer. Many languages have structures of variable and in C++ this structures is called class. Obviously this recent language brings improvement to these classes overall for the space management and creation of an object. The program uses the advantages to create nodes, elements and so on.

This part will explain the real advantages of the methods and how to apply it within finite element analysis.

#### Classes

The nodes, elements and materials description use the object technology of the C++ language. Globally, a node, an element, a material use the same properties, parameters to be defined, and these properties can be grouped in structures called classes.

The class is the mould of an object. Simple parameters and functions can belong to a class and are called respectively variables and methods

During the creation of an object in the program, we use a special function called constructor. It initialises the object e.g. the object Node has a constructor, which reads the number of the node and its coordinates.

We decided to create one class for each specific type of elements. Each class has its own variables and parameters, and the constructors are defined in conjunction with the specification of the elements.

In a problem, several elements are created, and an automatic procedure to create them is required. The technique used is called a dynamic pointers array.  Actually, at the start of the program is defined, for each type of elements, an array of pointers that has a size of 3000 addresses. The size is defined by default. When the computer detects a new element, it allocates memory for the new object (function new) and set a pointer in the array to the address of the class. Each variables or method from this object can be called by using this array giving the possibility to an automatic process.

The main part is reduced to several lines of code, becomes more readable and shows the developers’ interest for the encapsulation. The process to add a new class becomes much easier and requires adding the library and a couple of lines in the main program. The functions included in the class are just an interface i.e. the user doesn’t have to know how they work but what they need to work and supply them with the right information.

Finally, the subsequent work requires improvements and the developer will have to focus on the library rather than the global code because it makes the integration easier.

It is interesting to study an element class and see its structures.

For all of them, the variables are defined as private. That means that the only possibility to change them is to use the public methods created to manage these variables. The Public functions are the constructor to define the class, the “CalculateMatrix”  to define the local matrix and the “AssembleMatrix” to integrate the local matrix into the global Matrix.

#### LINK1

The link element is a simple bar; it has 2 nodes with 2 degrees of freedom per node. It is easy to set up because the program has to include the calculation of the local matrix.

Figure 3 : The element in the x-y plan. (adapted from [Error: Reference source not found])

The bar can be in another position than the horizontal position. The displacement has 2 components and that’s why the link element has four degree of freedom.

Figure 4 : Stiffness matrix for a link element. K is the general stiffness, s and c the sinus and cosine of the angle  (adapted from [Error: Reference source not found])

The class includes variables and methods. The developer has the right to use only methods because they are public. The method will handle the variables to calculate the length of the element, the local matrix, the angle of the slope, using the material, the node. The constructor defines most of these values. This function reads the data files respecting the format of the line for this type of element.

#### BEAM3

A beam element has 3 real degrees of freedom at each node i.e. displacements in x and y direction and rotation about z. To solve this case, the use of the theorem of superposition is required. Indeed, this is the sum of 2 basic case e.g. a link element and a bar with a DOF (degree of freedom) on the y axis and a DOF in rotation about z axis (see Figure 5).

Figure 5 : 2 basic cases (adapted from [Error: Reference source not found])

Figure 6 : The stiffness matrix for a bar is the sum of the stiffness matrix of the basic cases.

The previous doesn’t take into account the slope of the bar. It explains only how the cases are summed up.

The final result implemented in the program is function of the angle of inclination of the bar and it is just one matrix. The local matrix is determined by this theorical expression.

#### The mixed between Link1 and Beam3

In many cases, the mix of elements is possible. For example, the final user of SOFEA can use at the same time an element bar and an element link.

The program should treat these cases carefully since it causes problems because of the different number of degrees of freedom per element.

The size of the global matrix is not the product of the number of node by the number of degree of freedom per node.

This procedure makes the program more complex and some special method has been developed.

#### PLANE 42 (See Appendix 3: Class example)

The formulation of an element :

Shape functions are functions, which link the displacement of any point in the structure to the displacement of the node. There are as many functions as degree of freedom in an element.

The displacement of point of the element is a linear combination of the nodal displacement where their intensity are the values of the shape function in this point. Also the shape function has to be set up in respect with continuity.

To describe the behaviour of the element, displacements are represented by polynomial functions.

Remark : this method works with any kind of element (e.g. bar, plane).

The plane element is a simple rectangular element with 4 nodes. The development must be linear in x and y. The only possible choice is :

u(x,y)=a0+a1x+a2y+a3xy

This is the same displacement for v(x,y) because each node has 2 degree of freedom.

A more complex element has more nodes and their position depends on which family they belong (e.g. Lagrange or Serendip).

The determinations of the shape functions of simple element with random coordinates are difficult to find out. But for particular element, functions are easy to determine.

These specific elements, describe with a particular coordinate system are reference element. Their role in practical is important because with their help, the computer is able to calculate the stiffness characteristics of the real elements, which correspond to them.

Isoparametric element :

The isoparametric system has specific coordinates (,,), which lie between –1 and 1.

For a beam element, -1 will correspond to one end, 0, the middle of the beam and 1 to the other end.

Each coordinate can be written in function of (,,). The transformation must be bijective : it means that one coordinate correspond to another and vice versa.

For the calculation of the stiffness matrix, the expression of the shape element must be a function of ,. It is a simple change of coordinate, then for this operation the Jacobian matrix has to be computed. The derivates are mathematically known and the calculation of the stiffness matrix for a 2D element is nothing else than :

K=t−11−11BT(ζ,η)HB(ζ,η)det(J)dζdη

The formula can be generalised for 3D purpose. With the help of the change of system, the calculation of the stiffness matrix seems to process on the reference element and not with the real element.

The integration requires specific techniques to compute it. Actually the computer cannot solve this operation without special algorithms.

Remark :

With the previous hypothesis, if the shape of the real element is really different than the shape of the reference element, the approximation becomes bad and the risks to have a wrong result increase. The final user has to be careful about this.

“Certain element distortion cannot be tolerated and most commercial software will terminate solution (strictly element stiffness formulation) if one of these occur. Otherwise, commonly occurring types element distortion - aspect ratio, taper ratio and shape angle (skew angularity). How significantly these kinds of distortion affect the solution depend on many factors: how refined the mesh is, the position of the element in the model etc. however, over the years empirical rules for acceptable distorted shapes have been developed. In general, element distortion is found to affect low order elements much more than high order elements, due to the inability of the element shape functions to adequately account for complex deformation modes; in particular shear and bending.

Commonly accepted limits on such distortions are tabulated below for linear (with extra shape functions) and quadratic elements.”

A function has been developed to avoid the risks to have twisted element. Even if the risks of having a twisted element are low when the final user utilizes a commercial software, the risk becomes greater when the data file is hand made. Also this algorithm avoids the error by detecting and correcting it automatically. It works with 2 vectorial products and depending the sign of these products, the computer knows how the nodes are located in the plan.

Integration n:

The equivalent process in order to evaluate an integration is to make a balanced sum. Many research were done on this part and the most techniques can be quoted : Radau and Hammer for triangle and Newton-Cotes or Gauss for other element.

For a 2D element, the formula is :

I=f(ζ,η)dζdη=ijwiwjf(ζi,ηj)

Gauss method is the general method, which demand the minimum evaluation of the f function (the function case in our case is the expression of the stiffness matrix). The other methods give the same accuracy and with less point, but they are adapted to other elements and can’t be use in the general case.

The problem of integration is to find a compromise between the error and the time of computation. It is not true to think that increasing the number points for the integration, increases the accuracy of the final result.

For our plane element, the Gauss method has been chosen. There are 4 points of integration.

| Order 2 | I = ±13 | Weight Factor : 1 |
| --- | --- | --- |

array 1 : The array defines the coordinates of the 4 gauss point.

Figure 7 : Position of Gauss element in a plane element with 4 nodes.

A special function is designed to calculate the stiffness matrix. The flow chart for the integration is displayed in the Appendix 7 : Isoparametric.

The algorithm has been verified. The handling of such functions and such matrices is complicated because of their size. To be sure of the result obtained, MathCAD did the calculation of each matrix. The role of the developer was to use the debugger for one element and compare the result found by SOFEA and the result with MathCAD. Checking results with MathCAD also allowed to understand the process better and to correct the algorithm. The solution is efficient and reliable.

#### Material class

For a single problem, a part can use several materials, each material has properties.

The role of this class is to make a database, from the text file or from the final user data, grouping all the parameter of the material.

Each material becomes an object. Formulas have been generalised to call any material.

For the concerns of the project, so far, this class was developed so much. The program needs a couple of variables such the Young’s modulus or the Poisson’s ratio.

Nevertheless, this class can be filled by other parameters if the developer needs it. To do it, the user can fill the class directly in the sofcla header or make an inheritance from the material class and add his own parameters.

### The Master matrix assembly

In order to assemble the global matrix, “describing” the behaviour of the structure, we need to calculate, element by element, a local matrix for the behaviour of one element.

For the sake of simplicity, mechanic problems involving forces, displacement and stiffness were only the aim of this work. We can therefore speak only of stiffness matrix. However, we can include in the code classes taking care of heat transfer problems and so on.

Each element class have a function calculating the local stiffness matrix. The calculation changes with the number of degrees of freedom and different parameters of the element taken into account e.g. a plane element or a bar element.

For a plane element, the stiffness matrix was calculated with isoparametric coordinates, because of the shape of the element.

Once the local matrix has been calculated, it has to be assembled in the global stiffness matrix (Figure 8 and Figure 9).

Figure 8 : It shows two matrices which are respectively for 2 elements

The global stiffness matrix size is defined by the sum of the degree of freedom for each node. The matrix is a square matrix. The number of element inside doesn’t increase linearly but to the power of 2. It explains that this procedure has to be well-made otherwise problems will occur with large matrix.

Figure 9 : This is the master made from the 2 previous matrix. The operation is an addition.

Also it is impossible to define the size of such matrix before reading all the nodes and elements, because the number of degrees of freedom can change if several types of element are mixed.

Then global stiffness matrix, which is only a 2D array, is created with the dynamic array method. This method is simplified with the use of objects and pointers of pointers.

This method is appreciable, because the array is fit exactly to the need of each case, easy to set up and to be maintained, and take all the advantage of the object-oriented method.

When the Global Matrix is constructed, the assembly process can begin.

A node can belong to several elements and to be linked with different element means that its behaviour is affected by all of them.

Then to obtain the global matrix, we have to locate the local matrix in it using the number of the node and its degree of freedom. And once this unconditional step fulfilled, we add to the global matrix, all the local matrices scattered.

After the filling process finished, the global matrix is not ready yet to perform in the solving process for the reason that the boundary conditions are not defined. The displacements can be included in the global matrix. If a node is fixed in x or y direction, the modelling of the connection can be an element with a very high stiffness (see Figure 10). The discrete displacement of node linked on the body will be so small that it will be considered as equal to zero. This method is called the penalty method.

Figure 10 : Principle of the penalty method involving a large stiffness coefficient for a prescribed displacement.

### The matrix solving

For each degree of freedom, there is an equation. The computer has to solve large systems of x equation and x unknowns. The most popular way to do it is to use the LU decomposition because it is the method, which performs the best. The problem is that the matrix is too big and doing unnecessary operation, that’s why the band matrix solver will be investigated, wastes the processor time.

Once the global matrix is ready, the program has to solve the system of linear equations :

F=Kδ

This system involves a large number of equations which depends on the number of nodes and number of degrees of freedom and can reach thousands equations for a simple problem. Also the manipulation of such a matrix takes a lot of processors resources. This process must be treated with rigor and efficiency.

Rather than solve the system by Cramers’ Rule (a very basic method), the LU decomposition and the back substitution are preferred.

The stiffness matrix is transform in 2 triangular matrices :

K andL.U.δ=F

The computer solve first

L.X=F

and  finally obtain the displacement :

U.δ=X

After testing the program on a PC-350Mhz , the time for solving a relatively small case has been too long. The size of the matrix makes the process too long, even if the method for solving the equation is the best.

The analysis of the global matrix show that most of the variables inside the array are equal to zero i.e. there is no relation between all the nodes. If the numbering during the meshing process has been done correctly, the global matrix can remain roughly a diagonal matrix. The size of this diagonal will state the width of the new matrix. It will be a rectangular matrix instead of a square one. The number of elements inside will decrease considerably and so the number of operation. The solving time will decrease also. This method is called the skyline or banded matrix method. The process can be improved by adding the LU decomposition methods in the band solver.

### Matrix solvers

#### Gauss elimination

The first method found to solve the system of equation was the Gauss-Jordan elimination.

This method has an efficiency proved i.e. the method is quite stable as any other direct method and this is due to the way how the algorithm pivot. Actually, the full pivoting, the most recommended method consists to interchange rows and columns.

The method requires all the right hand sides to be stored and manipuled at the same time.

This is not the method chosen to solve the matrix in our program for different reason. Globally, the method is three times slower than the standard method. Also it is susceptible to round off error. These 2 disadvantages are fatal because the program requires a fast accurate method of solution.

Nevertheless the method is interesting to use if the matrix is triangular. It is called Gaussian Elimination with back substitution.

The matrices a and b have known values.

The pivoting operation starts from the line 4 and go forward. The algorithm is much more easy. In fact, the matrix is reduced to roughly the half. The computer will take into account only the non-zero number and by consequences reduce the cumber of calculations.

We will prefer to use the LU decomposition method rather than Gauss elimination, because it doesn’t have the disadvantage to know the right hand side in advances.

#### LU decomposition method

The LU decomposition consists to calculate two triangular matrices and proceeds to forward and back substitutions.

We suppose that there exists 2 triangular matrices U and L and their product is equal to A :

L.U=A

For the case of 4x4 matrices, we obtained :

Breaking the system of linear set into 2 successive equations give a solution easier to find because of solving a quite trivial triangular set of equations. We will use a forward substitution and then a back substitution.

To perform the LU decomposition, it is necessary to remind the general term for the sum :

αi1×β1j+...=aij

For all the case, it is always possible to use lii = 1. The calculation is then simplified.

The use of Crout’s algorithm arranges the equation in a certain order. If this algorithm is studied properly, one can remark that the aij terms is used only once and never again. It brings a real advantage in our case because it avoids us to use 2 big arrays.

Crout’s algorithm replace directly in the global matrix the lower and upper terms. The lower diagonal is not stored because the values of each term are equal to one.

#### Other methods

If the matrix is displayed, the number of null values is very high. The reason is that the nodes are not interconnected with each other but just with a couple of them. The sparsity must be exploited in order to economize on computer storage space and running time.

The most common method using this principle is the skyline method or known as the banded matrix.

The numbering of the node is done in such a specific way that numbers make diagonal, moving the zero values to the upper-right and lower-left edge of the matrix. Therefore, a rectangular matrix can be constituted ; its width is the width of the diagonal.

Figure 11 : Principle of the skyline method.

Arranging the matrix this way will allow the LU decomposition to work faster, because of less variables to process.

The company Algor proposes an algorithm really efficient to solve very large sets of equations. The technique is worthy when the problem has 100000 equations to solve (see Appendix 5 : new FEA frontier).

### Result output

A post-processor has not been developed for this code. The result cannot be displayed graphically. Nevertheless, the solving process gives the displacement of all the degree of freedom of each node. We decide to sort them and display the biggest displacement and the node number to give an idea where it is located.

The maximum displacement is given by :

DMX=u²+v²+w² where DMX is the maximum displacement, u the displacement on x axis, v the displacement y axis, w the displacement z axis.

The calculation of stress was first done with the strain but with the discrete displacement :

=H=HBq=Tq

T is called stresses matrix and allowed the computer to calculate stress from the displacement at the node. The calculation of the stress depends on the type of element and the formula =H should be preferred because it is light in computation.

The program calculates the discrete displacement in a structure. It reads a text file to take the information of the problem. This information is saved with a specific format defined by classes. The local matrix is made from these data for each element and then is located in the global matrix. The solution is found from the LU decomposition.

# Results

## Ansys tools

Ansys is used to compare the result found with SOFEA. To solve the same problem, the model should have the same data in input. The process adopted is to mesh with Ansys and put in a text file the coordinate of the node and the element properties, which can be read by SOFEA.

## Case studies

The program was not finished until we are sure that it gave accurate results. So we proceed to several tests. All types of elements were tested e.g. bar problems, beam problems, frame structures but in this report, the most relevant test is analysed and presented.

A load is applied on the middle.

Materials properties E=207 GPa  and  = 0.3

We want to know the maximum displacements, and its accuracy with a different number of Nodes, the solving time, and compare them with other software.

The program SOFEA was tested on a PC with AMD2-350Mhz. For the last case, we use two computers to compare the effect of the processors’ speed. The second processor was a Pentium 3-1Ghz.

The operating system was Windows 98 and Windows 2000, respectively.

The programs have been compiled with the same software and come from the same version.

This case study has 4 parts.

Part A :

The part is meshed with 95 nodes and 72 elements. The PC-350MHz was used.

Part B :

The part is meshed with 162 nodes and 130 elements. The slow PC-350MHz was used.

Part C :

The part is meshed with 561 nodes and 500 elements. The slow PC-350MHz  was used.

Part D :

This is the same case as C but the PC-1GHz  was used.

## Results

| Case | DMX ANSYS (mm) | DMX Sofea (mm) | % error | Solving time for SOFEA |
| --- | --- | --- | --- | --- |
| A – 95 el. | 0,28996 | 0,2782 | 4.1% | Less than 1s |
| B – 162 el. | 0,30032 | 0,2914 | 3,0% | 2,6s |
| C – 561 el. | 0,3231 | 0,31628 | 2,1% | 4min01s |
| D – 561 el. | 0,3231 | 0,31628 | 2,1% | 36s |

Figure 12 : Table grouping the result of the experiment.

We can neglect the time to assemble the matrix except for the case C which is 1min and 50 seconds.

Figure 13 : Result found with ANSYS. The stress intensity is plotted.

## Analysis

The greater the number of nodes, the more accurate the results become. To validate quickly Ansys was taken as a reference and to compare the result in the best conditions, the same model was taken for the two softwares. The difference between the 2 results is in any case less than 5%.

Also, the result is more accurate if the number of nodes increases although the solving time increases exponentially.

For the case of a slow computer, it is not impossible to choose a large number of nodes for the mesh, and the user must be advised to optimise it.

And finally, the processor performance is definitely important because it decreases, for the example C and D, the solving time by 6.69. In the case of the project included stochastic analysis, the solving process could be done hundreds of times to obtain the results.

Basically, the process could be improved because the time for solving a small mesh is too long. The future improvement is to decrease the size of the matrix so to decrease the number of operation by using a banded solver.

Also, the choice of a parallel computer to fulfil the project is very good choice and has to be taken in consideration.

# Code improvements and future improvements

The improvement all along the project has been done. The code produced during the development had some errors. They were corrected with the help of the debugger.

Also some functions implemented present some disadvantage. A solution to this problem has been found.

## Debugging

This is a process where the developer searches errors (or bugs) in the program. The debug process is to run step by step the program in order to see if it behaves well.

Most of the software to develop has one. This is a vital program even if it is very slow to use. Developers must have a logical view to determine where the errors are and the debug program to locate them accurately.

## Errors due to the final users

Errors can appear when the program is running. No message will appear. These errors are due to the final user who uses the program to solve a problem.

The more common error occurs when the user does write the model file himself. Often, he makes errors because of forgetting a degree of freedom during the definition of boundary conditions, or a wrong definition for elements. This implies an error during the reading process.

Also when the computers read large files under windows and creating by ANSYS, the reading process doesn’t detect the character ‘end of file EOF’. A solution was found but has not been implemented in the program so far. The developers have to write this special character in the ANSYS Macro.

## Source code improvement

The ideal choice is to replace the LU decomposition process by another, which takes less processors operation. We remark that the bigger the matrix, the slower the solution. We would probably have to set up a banded solver, which decrease the size of the global matrix. In the meantime, we can take advantage of a c++ library for the band solver and use the techniques offered by the object-oriented methods.

If the file reading has to be improved, the computer can read and put in memory a part of the file read rather than make a lot of disks access. The implementation of a pre and post processor could be done by the use of program such as FEView (see Appendix 4 : FEView).

The reading of the text file does not cause problem when the file is quite small. When the file is not, it is difficult to find a file editor able to read and edit such file. The problem causes by the EOF character should be considered if the user wants to read big files. A little algorithm, adding this character at the end has to be developed.

## Classes implementation

The interest of the language is the facilities to implement other code in the main one. It is perfectly suitable to the need of the global project. So far the program is able to solve basic elements, however the implementation of more complex ones is possible. The developer has to develop his own class describing the element and defining it, adding this class by adding a header file. Finally, he has to make a few modifications in the main part e.g. defining the class, defining the reading, implement the function to assemble the global matrix. If the code is without errors, the implement process lasts no more than 5 minutes and because it is easy the risks of error is very low.

# Conclusion

All objectives of the project were fulfilled successfully. The program is able to run and gives solutions for bar, beam and plane stress.

The development was focused on the “art” of programming i.e. many efforts to make the code readable to allow a better comprehension for next developer.

In the same way, the use of object has taken an important proportion. Many works have been done to permit the simplification of the integration of new classes for elements. Therefore the main code, which frames the program, is easy to follow and understanding the flow of information is simplified.

Despite these, hard work remains to improve the manipulation of the model file. Some investigation could be done with FEView to see if this tool could help.

The most significant effort to provide is to develop a better process for solving the linear equations. The use of a banded matrix seems good, and has been a standard method.

This year project was a year of personal involvement. Many benefits have been drawn out, as an improvement in object-oriented programming. It is also the discovery of new methods of work based more on the research.

Moreover, the research done on finite element analysis and stochastic method  allowed to understand better these methods and the need of each method.

The work furnished was really interesting because further the research and the discovery of these methods, it was not only mechanical project but also computing problems. I appreciate pretty much the practical aspect of it.  The relationship with my supervisor allowed discussion of the work done and the critics made bring a lot of added values.

# Appendices

## Appendix 1: algorithm

This is the flow chart of the main program. It is presented on 3 pages.

The 2 first pages are about the reading process. The last one is the manipulation of the matrix.

Remark :  The flow chart is not up-to-date. The reading of the element Plane 42 is missing however it is easy to imagine what the programmer has to add to make it working.

Definitions :

Headers

Variables

File definition

If reading is available

While not ENDOF FILE

Read ? « NODE »

Save the number of Nodes in memory

Read ? « ELEM »

Save the number of Elements in memory

Read ?

« NBLOCK »

While not end block

Save the Nodes details

Read ? « NElem »

4

3

2

1

5

Read ? « ET »

New type of Elements

If element is LINK1 ?

Create an initialize an object Link1.

If element is BEAM3 ?

Create an initialize an object Beam3.

Read ? « Mat »

Read Material properties : ex, poisson

Read ? « BForces »

Read forces, X, Y, ROTZ

Read ? « BDis »

Read displacements, X, Y, ROTZ

4

While not end of Elem

5

3

2

Global Matrix building

Assemble the stiffness matrix in the Global matrix

Apply boundary conditions

Solving with LU decomposition method and a back subsitution

Display the displacement solution nodes by nodes

End of Process

1

Close the file

## Appendix 2 : the data file

This document has been created to allow the user to understand how the data reading process works.

It describes a data file and its functions.

NODE,4

Number of Nodes

ELEM,3

Number of Elements

NBLOCK

1,0,1000

Coordonates of point1 : #, X, Y

2,2000,0

3,1000,0

4,00,00

ENDNODEBLOCK

EBLOCK

ET,1

# of the element : here 1

3,1,10,1,2

Descrition of the element : #, material, cross area, Node connected 1, Node connected 2

ET,3

1,1,5000,1000000,3,2

#, mat,cross area, second moment of area, node, node

2,1,5000,1000000,4,3

ENDEBLOCK

MAT

K,207000

Young Modulus; v for poisson coefficient

EMAT

BForces

3,0,-200,0

Forces description : #node, fx, fy, momentz

ENDBForces

BDis

1,0,0

Forces description : #node, u, v, rotz ; 0 = blocked, 1=available ;

4,0,0,0

ENDDis

## Appendix 3: Class example

The following document shows the structure of a class. The class is separated in 2 two distinct parts e.g. : private and public entities.

In this two part, entities may be variable or methods(function).

The function which has the same name as the class is named as constructor. This specific function initalizes parameters. This function is very useful.

class Plane2

{

private:

long number;

long NbLink[3];			// 3 connections

long Mat;				// Material

double Thickness;			// Thickness of the element

double **StiffMat;		// Double pointer for stiffness matrix

void DefineNodeDOF(Node *DNodes);

public:

void CalculateLength(Node *DNodes, Material *DMat);

Plane2(FILE *file,long Num,Node *DNodes);		// Constructor

void Display(int m);

void AssembleMatrix(double **Global,int MaxDOF,Node *DNodes);

};

## Appendix 4 : FEView

This pictures show the interface which enables the program to have a graphic interface.

Pre-processor above.

Post processor above

## Appendix 5 : new FEA frontier

This document is picked from “Global design News”, april 2002, page 87.

It shows that the process to solve the system is the most important part and is always submissive to improvement.

## Appendix 6 : The Code

The listing is the code of sofea. This is the main part.

#include <string.h>

#include <stdio.h>

#include <stdlib.h>

#include <iostream.h>

#include <math.h>

#include "tools.h"

#include "sofcla.h"

void main (void)

{

// Objects Creation

FILE *file ;

Node *DNodes;

Link1 *DLink1[300];

Material *DMat;

DMat =new Material[2];

Plane2 *DPlane2 [3000];

Beam3 *DBeam3 [3000];

Plane42 *DPlane42 [3000];

// this choice is made by default, a procedure can design to select an other file

char filename[]="c:\\sofea\\data.txt", Command[100], *ptr;

long NbElems=0, NbNodes=0, Number, NbObjectCreated=0;

int inc,countlev1, *indx, MaxNode=2;

long count[256]={0},num; // to know how objects created per kind of element

double *Forces, max;

ptr = &Command[0];

printf("\nReading process ...");

if ((file =fopen(filename,"r")) != NULL)

{

while (!::EndOfFile)

{

NextWord(file,&Command[0]);

//To know how nodes is there

if (!strcmp ("NODE",Command) && !::EndOfFile)

{

NextWord(file,&Command[0]);

sscanf(Command,"%ld", &NbNodes);

DNodes =new Node[NbNodes+1];

}

//To know how nodes there is

if (!strcmp ("ELEM",Command) && !::EndOfFile)

{

NextWord(file,&Command[0]);

sscanf(Command,"%d", &NbElems);

}

// Coordonates of NODE

if (!strcmp ("NBLOCK",Command) && !::EndOfFile)

{

NextWord(file,&Command[0]);

inc=0;

while (strcmp("ENDNODEBLOCK",Command) && !::EndOfFile)

{

DNodes[inc].Init(file);

NextWord(file,&Command[0]);

inc++;

}

}

//Coordonates of ELEM

if (!strcmp ("EBLOCK",Command)&& !::EndOfFile)

{

NextWord(file,&Command[0]);

inc=0;

countlev1=0; // First use no element defined

while (strcmp("ENDEBLOCK",Command) && !::EndOfFile)

{

if (!strcmp("ET",Command))

{

NextWord(file,&Command[0]);

sscanf(Command,"%i", &countlev1);

NextWord(file,&Command[0]);

}

//Utilisation de CASE perhaps !...

// The object relating to element must be created and then deleted

// A simple bar : Link1 on Ansys

if (countlev1==1 && strcmp("ENDEBLOCK",Command) && !::EndOfFile)

{

DLink1[count[countlev1]] = new Link1(file, Number,DNodes);		// Replace by a constructor

count[countlev1]++;

NbObjectCreated++;

}

// Plane2 !

if (countlev1==2 && strcmp("ENDEBLOCK",Command) && !::EndOfFile)

{

DPlane2[count[countlev1]] = new Plane2(file, Number,DNodes);

count[countlev1]++;

NbObjectCreated++;

}

// beam3 !

if (countlev1==3 && strcmp("ENDEBLOCK",Command) && !::EndOfFile)

{

DBeam3[count[countlev1]] = new Beam3(file, Number,DNodes);

count[countlev1]++;

NbObjectCreated++;

}

if (countlev1==42 && strcmp("ENDEBLOCK",Command) && !::EndOfFile)

{

DPlane42[count[countlev1]] = new Plane42(file, Number,DNodes);

count[countlev1]++;

NbObjectCreated++;

}

NextWord(file,&Command[0]);

}

}

// Material data reading process

if (!strcmp ("MAT",Command)&& !::EndOfFile)

{

inc=0;

NextWord(file,&Command[0]);

while (strcmp("EMAT",Command) && !::EndOfFile)

{

if (!strcmp("K",Command))

{

NextWord(file,&Command[0]);

sscanf(Command,"%le", &DMat[0].ex);

}

if (!strcmp("v",Command))

{

NextWord(file,&Command[0]);

sscanf(Command,"%le", &DMat[0].poisson);

}

NextWord(file,&Command[0]);

}

}

// Forces data reading process

if (!strcmp ("BForces",Command)&& !::EndOfFile)

{

MaxNode=0;

//Calculation of the position and the maximum DOF

for (inc= 0; inc<NbNodes;inc++)

{

DNodes[inc].CumSum=MaxNode;

MaxNode += DNodes[inc].DOF;

}

Forces = new double [MaxNode];

for (inc=0; inc <MaxNode;inc++)

Forces[inc]=0;

inc=0;

NextWord(file,&Command[0]);

while (strcmp("ENDBForces",Command) && !::EndOfFile)

{

sscanf(Command,"%ld",&Number);

for (inc=0;inc < DNodes[Number-1].DOF;inc++)

{

NextWord(file,&Command[0]);

sscanf(Command,"%le",&Forces[DNodes[Number-1].CumSum+ inc]);

}

NextWord(file,&Command[0]);

}

}

// Displacement data reading process

if (!strcmp ("BDis",Command)&& !::EndOfFile)

{

inc=0;

indx = new int [MaxNode];

for (inc=0; inc <MaxNode;inc++)

indx[inc]=1;

NextWord(file,&Command[0]);

while (strcmp("ENDDis",Command) && !::EndOfFile)

{

sscanf(Command,"%ld",&Number);

for (inc=0;inc < DNodes [Number-1].DOF;inc++)

{

NextWord(file,&Command[0]);

sscanf(Command,"%i",&indx[DNodes[Number-1].CumSum + inc]);

}

NextWord(file,&Command[0]);

}

}

}

printf("completed!\nGlobal Matrix building...");

// Global matrix creation ! square matrix for the moment

double **Global = new double*[MaxNode];

for (inc=0; inc < MaxNode; inc++)

Global[inc] = new double[MaxNode];

// Initialization of Global Matrix

for (inc=0;inc <MaxNode;inc++)

for (int inc2=0; inc2<MaxNode; inc2++)

Global[inc][inc2]=0;

// Assemble Matrix

for (inc=0;inc<count[1];inc++)

{

DLink1[inc]->CalculateLength(DNodes, DMat);

DLink1[inc]->AssembleMatrix(Global,MaxNode,DNodes);

}

for (inc=0;inc<count[2];inc++)

{

DPlane2[inc]->CalculateLength(DNodes, DMat);

DPlane2[inc]->AssembleMatrix(Global,MaxNode,DNodes);

}

for (inc=0; inc<count[3];inc++)

{

DBeam3[inc]->CalculateLength(DNodes, DMat);

DBeam3[inc]->AssembleMatrix(Global,MaxNode,DNodes);

}

for (inc=0; inc<count[42];inc++)

{

DPlane42[inc]->CalculateMatrix(DNodes, DMat);

DPlane42[inc]->AssembleMatrix(Global,MaxNode,DNodes);

}

// It is a "diagonal matrix" so to implement Ks only on the diagonal so far

// implementation of Ks function of the Maximum degree of freedom to locate the Node

for (inc=0;inc<MaxNode;inc++)

if (indx[inc] == 0)

Global[inc][inc]=1e200;

/*

cout<<"\n"<<"Global matrix"<<"\n";

// Globlal Matrix display

for (inc=0; inc<MaxNode;inc++)

{

for (int inc2=0; inc2 <MaxNode ; inc2++)

cout<<Global[inc][inc2]

<<"\t";

cout<<"\n";

}

*/

printf("completed!\nSolving by LU decomposition...");

// Solving part______________________________________________________________________________

//display the Result the backsubstition put all the result in the Forces vector..

//(unfortunatly deleting them)

// First step use the tool header and the Lower Upper Decomposition

ludcmp(Global,MaxNode,indx, Forces);

// Then let's do a back substitution to get the result

lubksb(Global,MaxNode,indx, Forces);

printf("completed!\nDisplacements sorting process...");

max=0;

for (inc=0;inc < NbNodes;inc++)

{

//	cout<<"\n Node "

//		<< inc+1;

//	for (int inc2=0;inc2 < DNodes[inc].DOF;inc2++)

//	{

//		cout<<"\n"

//			<<Forces[DNodes[inc].CumSum+ inc2];

//	}

if ((sqrt(pow(Forces[DNodes[inc].CumSum],2)+pow(Forces[DNodes[inc].CumSum+1],2))) > max)

{

max=sqrt(pow(Forces[DNodes[inc].CumSum],2)+pow(Forces[DNodes[inc].CumSum+1],2));

num=inc+1;

}

}

printf("completed!\n");

cout<<"\nThe Greater displacement is : "<<max

<<" and the Node number of the greater element is : "<<num;

cout<<"\nux = "<< Forces[DNodes[num-1].CumSum]

<<" and uy = "<<Forces[DNodes[num-1].CumSum+ 1];

cout<<"\nThis problem includes "<<NbNodes<<" nodes and "<<NbElems<<" elements.\n";

fclose (file);

}

}

The listing is the code of sofcla. This header describe the class and other useful function.

//#include "tools.h"

// sofcla.h

// headers to define all the class use in sofea.cpp

int EndOfFile=0;

//#define PI 3.14159265359

void NextWord(FILE *file, char *ptr)// what is the next word/Number

{

int Letter,inc=0;

Letter = fgetc(file);

while ((Letter != (int)'\n' && Letter!='\t' && Letter != ','&&Letter != ' '|| inc==0)&&Letter!= EOF)

{

if ((Letter == (int)'\n' || Letter=='\t' || Letter == ','||Letter == ' ') && inc==0)

inc=0;

else

if (Letter != (int)'\n' && Letter!='\t' && Letter != ','&&Letter != ' '&&Letter!= EOF)

{

*ptr=(char)Letter;

ptr++;

inc++;

}

Letter=fgetc(file);

if (Letter== EOF)

::EndOfFile=1;

}

*ptr='\0';

}

void SortNumber(long *NbLink,int index)

{

int Temp;

if ((index-1)!= 0) // break the recursivity

{

SortNumber (&NbLink[1], index-1);

for(int inc=0;inc<index-1;inc++)

if (NbLink[inc]>NbLink[inc+1])

{

Temp=NbLink[inc];

NbLink[inc]=NbLink[inc+1];

NbLink[inc+1]= Temp;

}

else

inc=index;

}

}

//-----------NODE--------------------------------------------------------

class Node

{

public:			// Caution No private because couldn't use some other function!!

// So to change

long number;			// What's the node's name ?

double nx, ny;				// Nodes Coordonates

long DOF;				// Number of DOF for this element

long CumSum;			// "Adress" in the vectors.

void Node::Init(FILE *file);

};

void Node::Init(FILE *file)		// function to initialize data during the file reading

{

char Command[100];

sscanf(Command,"%ld",&number);

NextWord(file,&Command[0]);

sscanf(Command,"%le", &nx);

NextWord(file,&Command[0]);

sscanf(Command,"%le", &ny);

DOF =0; CumSum=0;

}

//----------NODE----------------------------------------------------------

class Material

{

public:

long number;				// What's the Material's name ?

double ex, poisson;		// Material properties...

// ex = Young modulus on x

// poisson = Poisson Coef

// put the constructor to create the class

};

//-----------LINK1---------------------------------------------------------

class Link1					// Ansys code for 2D-spar

{

private:

long number;				// What's the element's name ?

long NbLink[2];			// define how many connection  And see alla bout the array!!

long Mat;				// Material

long R;					// Real

double Angle;	// Angle between the x-axis and the bar

double Length;	// lenght of the bar

double StiffMat[4][4];// Stiffness Matrix dim = 2

public:

void CalculateLength(Node *DNodes, Material *DMat);

Link1(FILE *file,long Num,Node *DNodes);

void Display(int m);

void AssembleMatrix(double **Global,int MaxDOF,Node *DNodes);

void DefineNodeDOF(Node *DNodes);

};

void Link1::CalculateLength(Node *DNodes, Material *DMat)

{

double CosA, SinA;

Length=sqrt(pow(((DNodes[(NbLink[0] - 1)].nx)-(DNodes[(NbLink[1] - 1)].nx)),2)+pow((DNodes[(NbLink[0] - 1)].ny)-(DNodes[(NbLink[1] - 1)].ny),2));

CosA=((DNodes[(NbLink[0] - 1)].nx)-(DNodes[(NbLink[1] - 1)].nx))/Length;

SinA=((DNodes[(NbLink[0] - 1)].ny)-(DNodes[(NbLink[1] - 1)].ny))/Length;

Angle = acos(CosA)*180/PI;

StiffMat[0][0]= DMat[(Mat - 1)].ex/Length*pow(CosA,2);

StiffMat[0][1]= DMat[(Mat - 1)].ex/Length*SinA*CosA;

StiffMat[0][2]= -StiffMat[0][0];

StiffMat[0][3]= -StiffMat[0][1];

StiffMat[1][0]= StiffMat[0][1];

StiffMat[1][1]= DMat[(Mat - 1)].ex/Length*pow(SinA,2);

StiffMat[1][2]= -StiffMat[0][1];

StiffMat[1][3]= -StiffMat[1][1];

StiffMat[2][0]= -StiffMat[0][0];

StiffMat[2][1]= -StiffMat[0][1];

StiffMat[2][2]= StiffMat[0][0];

StiffMat[2][3]= StiffMat[0][1];

StiffMat[3][0]= -StiffMat[0][1];

StiffMat[3][1]= -StiffMat[1][1];

StiffMat[3][2]= StiffMat[0][1];

StiffMat[3][3]= StiffMat[1][1];

}

Link1::Link1(FILE *file, long Num,Node *DNodes)		// function to initialize data during the file reading

{

char Command[100];

number = Num;

NextWord(file,&Command[0]);

sscanf(Command,"%ld",&Mat);

NextWord(file,&Command[0]);

sscanf(Command,"%ld", &R);

NextWord(file,&Command[0]);

sscanf(Command,"%ld", &NbLink[0]);

NextWord(file,&Command[0]);

sscanf(Command,"%ld", &NbLink[1]);

SortNumber (&NbLink[0],2);

DefineNodeDOF(DNodes);

}

void Link1::Display(int m)

{

cout<<"\n Element : "

<< m +1

<<"\n";

for (int inc=0; inc<4;inc ++)

{

for (int in=0;in<4; in++)

cout <<StiffMat[inc][in] <<"      ";

cout <<"\n";

}

}

void Link1::AssembleMatrix(double **Global, int MaxDOF,Node *DNodes)

{

// Function which assemble the Matrix !

for (int inc=0;inc<2; inc++)			//step for column (matrix 2x2)

for (int inc2=0; inc2<2; inc2++)		// step for row (matrix 2x2)

for(int inc3=0;inc3<2;inc3++)	// step for line

for (int inc4=0;inc4<2; inc4++)

Global[(DNodes[(NbLink[inc]-1)].CumSum+inc3)][(DNodes[NbLink[inc2]-1].CumSum+inc4)]+=StiffMat[(inc*2)+inc3][(inc2*2)+inc4];

}

void Link1::DefineNodeDOF(Node *DNodes)

{

for (int inc=0;inc<2;inc++)

if (!(DNodes[(NbLink[inc]-1)].DOF>2))

DNodes[(NbLink[inc]-1)].DOF=2;

}

//--------Beam3-----------------------------------------------------------------------

class Beam3

{

private:

long number;				// What's the element's name ?

long NbLink[2];			// define how many connection  And see alla bout the array!!

long Mat;				// Material

long R;					// Real

double EL;				// EI/L

double Area;			// Cross area

double Izz;				// inertia moment about z-axis

double Angle;	// Angle between the x-axis and the bar

double Length;	// lenght of the bar

double StiffMat[6][6];// Stiffness Matrix dim = 3 (3*2Nodes)

public:

void CalculateLength(Node *DNodes, Material *DMat);

Beam3(FILE *file,long Num,Node *DNodes);

void Display(int m);

void AssembleMatrix(double **Global,int MaxDOF,Node *DNodes);

void DefineNodeDOF(Node *DNodes);

};

void Beam3::CalculateLength(Node *DNodes, Material *DMat)

{

double CosA, SinA;

double A, B, K, M, F, G, P, H, Q,Z; // some variables

Length=sqrt(pow(((DNodes[(NbLink[0] - 1)].nx)-(DNodes[(NbLink[1] - 1)].nx)),2)+pow((DNodes[(NbLink[0] - 1)].ny)-(DNodes[(NbLink[1] - 1)].ny),2));

CosA=((DNodes[(NbLink[0] - 1)].nx)-(DNodes[(NbLink[1] - 1)].nx))/Length;

SinA=((DNodes[(NbLink[0] - 1)].ny)-(DNodes[(NbLink[1] - 1)].ny))/Length;

Angle = acos(CosA)*180/PI;

EL=DMat[(Mat - 1)].ex/Length;	// E/L ratio Young modulus over the length

//Previous Calculations : Check them 'cos I'm not sure of the result...

A=4*EL*Izz;				//A=Area Pense que ce n'est pas bon du tout;

Z=Area*EL;	// Verify this is the area use instead of A

B=2*EL*Izz; K=12*EL*Izz/Length/Length;

M = 6*EL*Izz/Length;

F= Z*pow(CosA,2)+ K*pow(SinA,2);

G= (Z-K)*CosA*SinA;

P = Z*pow(SinA,2)+ K*pow(CosA,2);

H = -M*SinA;

Q= M*CosA;

StiffMat[0][0]= F;

StiffMat[0][1]= G;

StiffMat[0][2]= H;

StiffMat[0][3]= -F;

StiffMat[0][4]= -G;

StiffMat[0][5]= H;

StiffMat[1][1]= P;

StiffMat[1][2]= Q;

StiffMat[1][3]= -G;

StiffMat[1][4]= -P;

StiffMat[1][5]= Q;

StiffMat[2][2]= A;

StiffMat[2][3]= -H;

StiffMat[2][4]= -Q;

StiffMat[2][5]= B;

StiffMat[3][3]= F;

StiffMat[3][4]= G;

StiffMat[3][5]= -H;

StiffMat[4][4]= P;

StiffMat[4][5]= -Q;

StiffMat[5][5]= A;

// Matrix Symmetry

for (int inc=0; inc<6;inc++)

for (int inc2=inc+1; inc2<6;inc2++)

StiffMat[inc2][inc]=StiffMat[inc][inc2];  // coordinate with the definition

}

Beam3::Beam3(FILE *file, long Num,Node *DNodes)		// function to initialize data during the file reading

{

char Command[100];

number = Num;

NextWord(file,&Command[0]);

sscanf(Command,"%ld",&Mat);				// read the material number

NextWord(file,&Command[0]);

sscanf(Command,"%le", &Area);			// read the area

NextWord(file,&Command[0]);

sscanf(Command,"%le", &Izz);			// read the moment of inertia

NextWord(file,&Command[0]);

sscanf(Command,"%ld", &NbLink[0]);		// Stiffness matrix

NextWord(file,&Command[0]);

sscanf(Command,"%ld", &NbLink[1]);

SortNumber (&NbLink[0],2);

DefineNodeDOF(DNodes);

}

void Beam3::Display(int m)

{

cout<<"\n Element : "

<< m +1

<<"\n";

for (int inc=0; inc<6;inc ++)

{

for (int in=0;in<6; in++)

cout <<StiffMat[inc][in] <<"      ";

cout <<"\n";

}

}

void Beam3::AssembleMatrix(double **Global, int MaxDOF,Node *DNodes)

{

// Function which assemble the Matrix !

for (int inc=0;inc<2; inc++)			//step for column (4 matrices 3x3)

for (int inc2=0; inc2<2; inc2++)		// step for row (4 matrices 3x3)

for(int inc3=0;inc3<3;inc3++)	// step for line (row)

for (int inc4=0;inc4<3;inc4++)// step for line (column)

Global[(DNodes[(NbLink[inc]-1)].CumSum+inc3)][(DNodes[NbLink[inc2]-1].CumSum+inc4)]+=StiffMat[(inc*3)+inc3][(inc2*3)+inc4];

}

void Beam3::DefineNodeDOF(Node *DNodes)

{

for (int inc=0;inc<2;inc++)

if (!(DNodes[(NbLink[inc]-1)].DOF>3))

DNodes[(NbLink[inc]-1)].DOF=3;

}

//--------Plane2----------------------------------------------------------------------

//Comment on the Matrices used later on

//

//

//

//

//

//

//

//

//

//-Plane 2---------------------------------------------------------------------------

class Plane2

{

private:

long number;

long NbLink[3];			// 3 connections

long Mat;				// Material

long R;					// Real

double EL;				// EI/L

double Area;			// Cross area

double Izz;				// inertia moment about z-axis

double Angle;			// Angle between the x-axis and the bar

double Length;			// lenght of the bar

double Thickness;		//Thickness of the element

double **StiffMat;		//

double ElemArea;		//Element's Area..

void DefineNodeDOF(Node *DNodes);

public:

void CalculateLength(Node *DNodes, Material *DMat);

Plane2(FILE *file,long Num,Node *DNodes);

void Display(int m);

void AssembleMatrix(double **Global,int MaxDOF,Node *DNodes);

};

void Plane2::CalculateLength(Node *DNodes, Material *DMat)

{

double *col, *d, twodet;

int *indx,inc2, inc,indxx1,indxx2,indxy1,indxy2;

double DMatrixConst;

double **DMatrix=new double*[3];

for (inc=0; inc < 3; inc++)			// 3*3

DMatrix[inc] = new double[3];

// Calculation of D

//DMatrixConst

DMatrixConst=DMat[(Mat - 1)].ex/(1-pow(DMat[(Mat - 1)].poisson,2));

for (inc=0;inc<3;inc++)

for (inc2=0; inc2<3; inc2++)

DMatrix[inc][inc2]=0;

DMatrix[0][0]=DMatrixConst;

DMatrix[0][1]=DMat[(Mat - 1)].poisson*DMatrixConst;

DMatrix[1][0]=DMat[(Mat - 1)].poisson*DMatrixConst;

DMatrix[1][1]=DMatrixConst;

DMatrix[2][2]=0.5*(1-DMat[(Mat - 1)].poisson)*DMatrixConst;

//Calculation of the determinant

twodet=DNodes[(NbLink[1] - 1)].nx*DNodes[(NbLink[2] - 1)].ny-DNodes[(NbLink[2] - 1)].nx*DNodes[(NbLink[1] - 1)].ny-DNodes[(NbLink[0] - 1)].nx*DNodes[(NbLink[2] - 1)].ny+DNodes[(NbLink[0] - 1)].nx*DNodes[(NbLink[1] - 1)].ny+DNodes[(NbLink[2] - 1)].nx*DNodes[(NbLink[0] - 1)].ny-DNodes[(NbLink[1] - 1)].nx*DNodes[(NbLink[0] - 1)].ny;

// calculation of the local Matrix using the algorithms of the log book

for (inc=0; inc<3; inc++)

for (inc2=0; inc2<3; inc2++)

{

if (inc==0)

{

indxx1=3-1;

indxx2=2-1;

}

if (inc==1)

{

indxx1=1-1;

indxx2=3-1;

}

if (inc==2)

{

indxx1=2-1;

indxx2=1-1;

}

if (inc2==0)

{

indxy1=3-1;

indxy2=2-1;

}

if (inc2==1)

{

indxy1=1-1;

indxy2=3-1;

}

if (inc2==2)

{

indxy1=2-1;

indxy2=1-1;

}

for (int inc3=0;inc3<2;inc3++)

for (int inc4=0; inc4<2; inc4++)

{

if (inc3==0 && inc4==0)

StiffMat[inc*2+inc3][inc2*2+inc4]=Thickness/(2*twodet)*(DMatrix[0][0]*((DNodes[(NbLink[indxx2] - 1)].ny)-(DNodes[(NbLink[indxx1] - 1)].ny))*((DNodes[(NbLink[indxy2] - 1)].ny)-(DNodes[(NbLink[indxy1] - 1)].ny))+DMatrix[2][2]*((DNodes[(NbLink[indxy1] - 1)].nx)-(DNodes[(NbLink[indxy2] - 1)].nx))*((DNodes[(NbLink[indxx1] - 1)].nx)-(DNodes[(NbLink[indxx2] - 1)].nx)));

if (inc3==1 && inc4==0)

StiffMat[inc*2+inc3][inc2*2+inc4]=Thickness/(2*twodet)*(DMatrix[0][1]*((DNodes[(NbLink[indxy2] - 1)].ny)-(DNodes[(NbLink[indxy1] - 1)].ny))*((DNodes[(NbLink[indxx1] - 1)].nx)-(DNodes[(NbLink[indxx2] - 1)].nx))+DMatrix[2][2]*((DNodes[(NbLink[indxy1] - 1)].nx)-(DNodes[(NbLink[indxy2] - 1)].nx))*((DNodes[(NbLink[indxx2] - 1)].ny)-(DNodes[(NbLink[indxx1] - 1)].ny)));

if (inc3==0 && inc4==1)

StiffMat[inc*2+inc3][inc2*2+inc4]=Thickness/(2*twodet)*(DMatrix[0][1]*((DNodes[(NbLink[indxx1] - 1)].nx)-(DNodes[(NbLink[indxx2] - 1)].nx))*((DNodes[(NbLink[indxy2] - 1)].ny)-(DNodes[(NbLink[indxy1] - 1)].ny))+DMatrix[2][2]*((DNodes[(NbLink[indxx1] - 1)].nx)-(DNodes[(NbLink[indxx2] - 1)].nx))*((DNodes[(NbLink[indxy2] - 1)].ny)-(DNodes[(NbLink[indxy1] - 1)].ny)));

if (inc3==1 && inc4==1)

StiffMat[inc*2+inc3][inc2*2+inc4]=Thickness/(2*twodet)*(DMatrix[1][1]*((DNodes[(NbLink[indxx1] - 1)].nx)-(DNodes[(NbLink[indxx2] - 1)].nx))*((DNodes[(NbLink[indxy1] - 1)].nx)-(DNodes[(NbLink[indxy2] - 1)].nx))+DMatrix[2][2]*((DNodes[(NbLink[indxy2] - 1)].ny)-(DNodes[(NbLink[indxy1] - 1)].ny))*((DNodes[(NbLink[indxx2] - 1)].ny)-(DNodes[(NbLink[indxx1] - 1)].ny)));

}

}

}

Plane2::Plane2(FILE *file, long Num,Node *DNodes)		// function to initialize data during the file reading

{

char Command[100];

number = Num;

NextWord(file,&Command[0]);

sscanf(Command,"%ld",&Mat);				// read the material number

NextWord(file,&Command[0]);

sscanf(Command,"%le", &Area);			// read the area

NextWord(file,&Command[0]);

sscanf(Command,"%le", &Izz);			// read the moment of inertia

NextWord(file,&Command[0]);

sscanf(Command,"%le", &Thickness);		// read the thickness

for(int inc=0; inc < 3; inc++)

{

NextWord(file,&Command[0]);

sscanf(Command,"%ld", &NbLink[inc]);	// Stiffness matrix

}

StiffMat= new double*[6];

for (inc=0; inc < 6; inc++)					// 6*6

StiffMat[inc] = new double[6];

// Sort the number of Node

SortNumber (&NbLink[0],3);

DefineNodeDOF(DNodes);					// To read the boundary conditions

// as forces or displacements

}

void Plane2::Display(int m)

{

cout<<"\n Element : "

<< m +1

<<"\n";

for (int inc=0; inc<6;inc ++)

{

for (int in=0;in<6; in++)

cout <<StiffMat[inc][in] <<"      ";

cout <<"\n";

}

}

void Plane2::AssembleMatrix(double **Global, int MaxDOF,Node *DNodes)

{

// Function which assemble the Matrix !

for (int inc=0;inc<3; inc++)			//step for column (3 Nodes 2x2)

for (int inc2=0; inc2<3; inc2++)		// step for row (3 Nodes 2x2)

for(int inc3=0;inc3<2;inc3++)	// step for line (row)

for (int inc4=0;inc4<2;inc4++)// step for line (column)

{

Global[(DNodes[(NbLink[inc]-1)].CumSum+inc3)][(DNodes[NbLink[inc2]-1].CumSum+inc4)]+=StiffMat[(inc*2)+inc3][(inc2*2)+inc4];

inc4=inc4;

}

}

void Plane2::DefineNodeDOF(Node *DNodes)

{

for (int inc=0;inc<3;inc++)

if (!(DNodes[(NbLink[inc]-1)].DOF>2))

DNodes[(NbLink[inc]-1)].DOF=2;

}

//--------Plane 42--------------------------------------------------------------------

//Comment on the Matrices used later on

//

//

//

//

//

//

//

//

//

//-Plane 42--------------------------------------------------------------------------

class Plane42

{

private:

long number;

long NbLink[4];			// 4 connections

long Mat;				// Material

double Thickness;		//Thickness of the element

double **StiffMat;		//

void DefineNodeDOF(Node *DNodes);

void Sort(Node*DNodes);

public:

void CalculateMatrix(Node *DNodes, Material *DMat);

Plane42(FILE *file,long Num,Node *DNodes);

void AssembleMatrix(double **Global,int MaxDOF,Node *DNodes);

};

void Plane42::CalculateMatrix(Node *DNodes, Material *DMat)

{

double Coordonate[2],Jacobian [2][2],JacDet,Temp;

double EMatrixConst;

double RValue[4]={-.57735,.57735,.57735,-.57735};	// for 0,1,2 and 3

double SValue[4]={-.57735,-.57735,.57735,.57735};	// for 0,1,2 and 3

double NShFu[4];		// shape function

double NDerR[4];		// derivative relative to R

double NDerS[4];		// derivative relative to S

int inc,inc2, inc3, inc4, inc5;

double DMatrixConst;

double **EMatrix=new double*[3];

double **BMatrix=new double*[3];		//matrix B

double **BtMatrix=new double*[8];		// Transpose Matrix

double **TempMatrix=new double*[3];

double temp02[8][8];

Sort (DNodes);							// Position node in the right order

for (inc=0;inc<8;inc++)

for(inc2=0; inc2<8; inc2++)

temp02[inc][inc2]=0;

for (inc=0; inc < 3; inc++)			// 3*8

{

BMatrix[inc] = new double[8];

TempMatrix[inc] = new double[8];

}

for (inc=0; inc < 3; inc++)			// 3*3

EMatrix[inc] = new double[3];

for (inc=0; inc < 8; inc++)			// 8*3

BtMatrix[inc] = new double[3];

//quadrature order is 2

Coordonate[0]=1;

Coordonate[1]=-1;

//Calculatoin de EMatrix

EMatrixConst=DMat[(Mat - 1)].ex/(1-pow(DMat[(Mat - 1)].poisson,2));

for (inc=0;inc<3;inc++)

for (inc2=0; inc2<3; inc2++)

EMatrix[inc][inc2]=0;

EMatrix[0][0]=EMatrixConst;

EMatrix[0][1]=(DMat[(Mat - 1)].poisson*EMatrixConst);

EMatrix[1][0]=(DMat[(Mat - 1)].poisson*EMatrixConst);

EMatrix[1][1]=EMatrixConst;

EMatrix[2][2]=(0.5*(1-DMat[(Mat - 1)].poisson)*EMatrixConst);

StiffMat= new double*[8];

for (inc=0; inc < 8; inc++)					// 8*8

StiffMat[inc] = new double[8];

for (inc=0;inc < 4;inc++)			// where 2 is the order of the integration

{

// Definition of Ni, and their derivatives

// faux

NShFu[0]= 0.25*(1-RValue[inc])*(1-SValue[inc]);

NShFu[1]= 0.25*(1+RValue[inc])*(1-SValue[inc]);

NShFu[2]= 0.25*(1+RValue[inc])*(1+SValue[inc]);

NShFu[3]= 0.25*(1-RValue[inc])*(1+SValue[inc]);

NDerR[0]= -0.25*(1-SValue[inc]);

NDerR[1]= 0.25*(1-SValue[inc]);

NDerR[2]= 0.25*(1+SValue[inc]);

NDerR[3]= -0.25*(1+SValue[inc]);

NDerS[0]= -0.25*(1-RValue[inc]);

NDerS[1]= -0.25*(1+RValue[inc]);

NDerS[2]= 0.25*(1+RValue[inc]);

NDerS[3]= 0.25*(1-RValue[inc]);

// initialisation of the Jacobian Matrix

for (inc4=0; inc4<2; inc4++)

for (inc5=0; inc5<2; inc5++)

Jacobian[inc4][inc5]=0;

for(inc3=0;inc3<4;inc3++)

{

Jacobian[0][0]+=NDerR[inc3]*DNodes[(NbLink[inc3]-1)].nx;

Jacobian[0][1]+=NDerS[inc3]*DNodes[(NbLink[inc3]-1)].nx;

Jacobian[1][0]+=NDerR[inc3]*DNodes[(NbLink[inc3]-1)].ny;

Jacobian[1][1]+=NDerS[inc3]*DNodes[(NbLink[inc3]-1)].ny;

}

JacDet=Jacobian[0][0]*Jacobian[1][1]-Jacobian[1][0]*Jacobian[0][1];

// Replacement Of the jacobian matrix by its inverse

Temp = Jacobian[0][0]/JacDet;

Jacobian[0][0] = Jacobian[1][1]/JacDet;

Jacobian[1][1] = Temp;

Temp=Jacobian[1][0]/(-JacDet);

Jacobian[1][0] = Jacobian[0][1]/(-JacDet);

Jacobian[0][1] = Temp;

// Calculation of BMatrix[][]

for (inc3=0;inc3<3;inc3++)

for(inc4=0;inc4<8;inc4++)

BMatrix[inc3][inc4] = 0;

for (inc3=0;inc3<4;inc3++)

{

BMatrix[0][inc3*2]=Jacobian[0][0]*NDerR[inc3]+Jacobian[1][0]*NDerS[inc3];

BMatrix[1][inc3*2+1]=Jacobian[0][1]*NDerR[inc3]+Jacobian[1][1]*NDerS[inc3];

BMatrix[2][inc3*2]=BMatrix[1][inc3*2+1];

BMatrix[2][inc3*2+1]=BMatrix[0][inc3*2];

}

// Calculation of the Transpose of BMatrix[][]

matrix_transpose(BtMatrix,BMatrix,8,3);

for (inc3=0;inc3<3;inc3++)

for(inc4=0;inc4<8;inc4++)

BMatrix[inc3][inc4] = BMatrix[inc3][inc4]*JacDet*1;	// 1 for the weight

matrix_multiply(EMatrix, BMatrix, TempMatrix,3,3,8);

matrix_multiply(BtMatrix, TempMatrix, StiffMat,8,3,8);

for (inc3=0; inc3<8;inc3++)

for (inc4=0; inc4<8; inc4++)

{

temp02[inc3][inc4] +=StiffMat [inc3][inc4];

inc5=0;}

}

for (inc3=0; inc3<8; inc3++)

for(inc4=0; inc4<8; inc4++)

StiffMat [inc3][inc4]=temp02[inc3][inc4];

}

Plane42::Plane42(FILE *file, long Num,Node *DNodes)		// function to initialize data during the file reading

{

char Command[100];

number = Num;

NextWord(file,&Command[0]);

sscanf(Command,"%ld",&Mat);				// read the material number

//	NextWord(file,&Command[0]);

//	sscanf(Command,"%le", &Thickness);		// read the thickness

for(int inc=0; inc < 4; inc++)

{

NextWord(file,&Command[0]);

sscanf(Command,"%ld", &NbLink[inc]);	// Stiffness matrix

}

// Sort the number of Node

//SortNumber (&NbLink[0],6);

DefineNodeDOF(DNodes);					// To read the boundary conditions

// as forces or displacements

}

void Plane42::AssembleMatrix(double **Global, int MaxDOF,Node *DNodes)

{

// Function which assemble the Matrix !

for (int inc=0;inc<4; inc++)			//step for column (3 Nodes 2x2)

for (int inc2=0; inc2<4; inc2++)		// step for row (3 Nodes 2x2)

for(int inc3=0;inc3<2;inc3++)	// step for line (row) (2 DOF)

for (int inc4=0;inc4<2;inc4++)// step for line (column) (2 DOF)

{

Global[(DNodes[(NbLink[inc]-1)].CumSum+inc3)][(DNodes[NbLink[inc2]-1].CumSum+inc4)]+=StiffMat[(inc*2)+inc3][(inc2*2)+inc4];

inc4=inc4;

}

}

void Plane42::DefineNodeDOF(Node *DNodes)

{

for (int inc=0;inc<4;inc++)

if (!(DNodes[(NbLink[inc]-1)].DOF>2))

DNodes[(NbLink[inc]-1)].DOF=2;

}

void Plane42::Sort(Node*DNodes)

{

long Temp;

double vectorAB[2],vectorBC [2], vectorBD[2],vectorDA[2],vectorCD[2];

/*	A equiv 0

B equiv 1

C equiv 2

D equiv 3 */

// vector definition

vectorAB[0]=DNodes[(NbLink[0]-1)].nx-DNodes[(NbLink[1]-1)].nx;

vectorAB[1]=DNodes[(NbLink[0]-1)].ny-DNodes[(NbLink[1]-1)].ny;

vectorBC[0]=DNodes[(NbLink[1]-1)].nx-DNodes[(NbLink[2]-1)].nx;

vectorBC[1]=DNodes[(NbLink[1]-1)].ny-DNodes[(NbLink[2]-1)].ny;

//Is C at the right place

// Exchange C and B

if((vectorAB[0]*vectorBC[1]-vectorBC[0]*vectorAB[1])<0) // if =0 if ABC is straight

{

Temp=NbLink [1];

NbLink [1]=NbLink[2];

NbLink[2]=Temp;

}

//Is D at the right place

//CD^DA

vectorAB[0]=DNodes[(NbLink[0]-1)].nx-DNodes[(NbLink[1]-1)].nx;

vectorAB[1]=DNodes[(NbLink[0]-1)].ny-DNodes[(NbLink[1]-1)].ny;

vectorBD[0]=DNodes[(NbLink[1]-1)].nx-DNodes[(NbLink[3]-1)].nx;

vectorBD[1]=DNodes[(NbLink[1]-1)].ny-DNodes[(NbLink[3]-1)].ny;

vectorCD[0]=DNodes[(NbLink[2]-1)].nx-DNodes[(NbLink[3]-1)].nx;

vectorCD[1]=DNodes[(NbLink[2]-1)].ny-DNodes[(NbLink[3]-1)].ny;

vectorDA[0]=DNodes[(NbLink[3]-1)].nx-DNodes[(NbLink[0]-1)].nx;

vectorDA[1]=DNodes[(NbLink[3]-1)].ny-DNodes[(NbLink[0]-1)].ny;

if((vectorCD[0]*vectorDA[1]-vectorDA[0]*vectorCD[1])<0) // if =0 if ABC is straight

{

//AB^BD

if((vectorAB[0]*vectorBD[1]-vectorBD[0]*vectorAB[1])<0) // if =0 if ABC is straight

{

Temp=NbLink[1];						// temp takes B value

NbLink[1]=NbLink[3];				// B takes D value

NbLink[3]=NbLink[2];				// D takes C value

NbLink[2]=Temp;						// C takes B value

}

else

{

//exchange C and D

Temp=NbLink[2];					// temp takes C value

NbLink[2]=NbLink[3];			// C takes D value

NbLink[3]=Temp;					// D takes C value

}

}

}

The listing is the code of tools. This module is for mathematical functions applied to matrix manipulation. This module was designed again because the compatibility was wrong.

/**************/

/* tools.h  */

/**************/

#define TINY 1.0e-50

#define PI 3.14159265359

/*definitions for ran1()*/

#define M1 259200

#define IA1 7141

#define IC1 54733

#define RM1 (1.0/M1)

#define M2 134456

#define IA2 8121

#define IC2 28411

#define RM2 (1.0/M2)

#define M3 243000

#define IA3 4561

#define IC3 51349

/********************/

/* matrix utilities */

/********************/

/*********************************************************************/

/* The following routines are from nrutil.c (Numerical Recipes in C )*/

/*********************************************************************/

void nerror(char *error_text)

{

fprintf(stderr,"Run time error...\n");

fprintf(stderr,"%s\n",error_text);

fprintf(stderr,"Exiting to system...\n\n");

exit(1);

}

/****************************/

int *ivector(int nl,int nh)

{

int *v;

v=(int *)malloc((unsigned) (nh-nl+1)*sizeof(int));

if(!v) nerror ("allocation failure in ivector()");

return v-nl;

}

/****************************/

void free_ivector(int *v,int nl)

{

free((char *) (v+nl));

}

/****************************/

double *dvector(int nl,int nh)

{

double *v;

v=(double *)malloc((unsigned) (nh-nl+1)*sizeof(double));

if(!v) nerror ("allocation failure in dvector()");

return v-nl;

}

/******************************/

void free_dvector(double *v,int nl,int nh)

{

free((char *) (v+nl));

}

/****************************/

double **dmatrix(int nrl,int nrh,int ncl,int nch)

{

int i;

double **m;

m=(double **)malloc((unsigned) (nrh-nrl+1)*sizeof(double*));

if(!m) nerror("Allocation failure 1 in dmatrix()");

m -= nrl;

for(i=nrl;i<=nrh;i++)   {

m[i]=(double *) malloc((unsigned)(nch-ncl+1)*sizeof(double));

if(!m[i])nerror("Allocation failure 2 in dmatrix()");

m[i] -= ncl;

}

return m;

}

/******************************/

void free_dmatrix(double **m,int nrl, int nrh,int ncl,int nch)

{

int i;

for(i=nrh;i>=nrl;i--)   free((char *) (m[i]+ncl));

free((char *) (m+nrl));

}

/****************************/

float **fmatrix(int nrl, int nrh,int ncl,int nch)

{

int i;

float **m;

m=(float **)malloc((unsigned) (nrh-nrl+1)*sizeof(float*));

if(!m) nerror("Allocation failure 1 in fmatrix()");

m -= nrl;

for(i=nrl;i<=nrh;i++)   {

m[i]=(float *) malloc((unsigned)(nch-ncl+1)*sizeof(float));

if(!m[i])nerror("Allocation failure 2 in fmatrix()");

m[i] -= ncl;

}

return m;

}

/******************************/

void free_fmatrix(float **m,int nrl,int nrh,int ncl,int nch)

{

int i;

for(i=nrh;i>=nrl;i--)   free((char *) (m[i]+ncl));

free((char *) (m+nrl));

}

/******************************/

int **imatrix(int nrl,int nrh,int ncl,int nch)

{

int i;

int **m;

m=(int **)malloc((unsigned) (nrh-nrl+1)*sizeof(int*));

if(!m) nerror("Allocation failure 1 in imatrix()");

m -= nrl;

for(i=nrl;i<=nrh;i++)   {

m[i]=(int *) malloc((unsigned)(nch-ncl+1)*sizeof(int));

if(!m[i])nerror("Allocation failure 2 in imatrix()");

m[i] -= ncl;

}

return m;

}

/******************************/

void free_imatrix(int **m,int nrl,int nrh,int ncl,int nch)

{

int i;

for(i=nrh;i>=nrl;i--)   free((char *) (m[i]+ncl));

free((char *) (m+nrl));

}

/* solvers */

/********************************/

/* LU Decomposition of a matrix */

/* From Numerical Recipes in C  */

/********************************/

void ludcmp(double **a,long n,int *indx,double *d)

{

int i,imax,j,k;

double big,dum,sum,temp;

double *vv;

vv=dvector(1,n);

*d=1.0;

for(i=1;i<=n;i++)

{

big=0.0;

for(j=1;j<=n;j++) if((temp=fabs(a[i-1][j-1])) > big) big=temp;

if(big == 0.0) nerror("Singular matrix in routine LUDCMP");

vv[i]=1.0/big;

}

for(j=1;j<=n;j++)

{

for(i=1;i<j;i++)

{

sum=a[i-1][j-1];

for(k=1;k<i;k++) sum -= a[i-1][k-1]*a[k-1][j-1];

a[i-1][j-1]=sum;

}

big=0.0;

for(i=j;i<=n;i++)   {

sum=a[i-1][j-1];

for(k=1;k<j;k++) sum -= a[i-1][k-1]*a[k-1][j-1];

a[i-1][j-1]=sum;

if((dum=vv[i]*fabs(sum)) >= big)    {

big = dum;

imax=i;

}

}

if(j != imax)   {

for(k=1;k<=n;k++)   {

dum=a[imax-1][k-1];

a[imax-1][k-1]=a[j-1][k-1];

a[j-1][k-1]=dum;

}

*d = -(*d);

vv[imax-1]=vv[j-1];

}

indx[j-1]=imax;

if (a[j-1][j-1] ==0.0) a[j-1][j-1]=TINY;

if(j != n)  {

dum=1.0/(a[j-1][j-1]);

for(i=j+1;i<=n;i++) a[i-1][j-1] *= dum;

}

}

free_dvector(vv,1,n);

}

/**************************************/

void lubksb(double **a,long n,int *indx,double *b)

{

int i,ii=0,ip,j;

double sum;

for(i=1;i<=n;i++)   {

ip=indx[i-1];

sum=b[ip-1];

b[ip-1]=b[i-1];

if(ii)  for(j=ii;j<=i-1;j++) sum -= a[i-1][j-1]*b[j-1];

else if(sum) ii=i;

b[i-1] = sum;

}

for(i=n;i>=1;i--)   {

sum=b[i-1];

for(j=i+1;j<=n;j++) sum -= a[i-1][j-1]*b[j-1];

b[i-1]=sum/(a[i-1][i-1]);

}

}

/*****************************************/

void mprove(double **a,double **alud,int n,int *indx,double *b,double *x)

{

int i,j;

double sdp;

double *r;

r = dvector(1,n);

for(i=1;i<=n;i++)   {

sdp = -b[i];

for(j=1;j<=n;j++)   sdp += a[i][j]*x[j];

r[i] = sdp;

}

lubksb(alud,n,indx,r);

for(i=1;i<=n;i++)   x[i] -= r[i];

free_dvector(r,1,n);

}

/*****************************/

void invert_matrix(double **g, double **h,int n,int *indx, double *col,double *d)

{

int i,j;

/* Invert "g" matrix return result in "h" matrix*/

ludcmp(g,n,indx,d);

for(j=1;j<=n;j++)   {

for(i=1;i<=n;i++)   col[i-1]=0.0;

col[j-1]=1.0;

lubksb(g,n,indx,col);

for(i=1;i<=n;i++) h[i-1][j-1]=col[i-1];

}

}

/*****************************************/

void band_solver(double **a, double *c, int neq,int nbw)

{

int i,j,k,l,n;

int neqp,neqm,lim;

double dum;

/* Treat the case of one or more independent equations */

if(nbw == 1)    {

for(n=1;n<=neq;n++) c[n] /= a[n][1];

return;

}

neqp = neq+1;

neqm = neq-1;

/* Forward reduction of the coefficient matrix [A] */

for(n=1;n<=neqm;n++)    {

if(nbw > (neqp-n)) lim = (neqp-n); else lim = nbw;

for(l=2;l<=lim;l++) {

dum = a[n][l] / a[n][1];

if(dum != 0.0)  {   /* Modified to eliminate 0 mult.*/

i = n+l-1;

j = 0;

for(k=l;k<=lim;k++) {

j++;

a[i][j] -= (dum*a[n][k]);

}

a[n][l] = dum;

}

}

}

/* Forward reduction of the constant vector {c} */

for(n=1;n<=neqm;n++)    {

if(nbw > (neqp-n)) lim = (neqp-n); else lim = nbw;

for(l=2;l<=lim;l++) {

i = n+l-1;

c[i] -= (a[n][l] * c[n]);

}

c[n] /= a[n][1];

}

c[neq] /= a[neq][1];

/* Back substitution. Former unknowns {x} overwrite {c} */

for(n=neqm;n>=1;n--)    {

if(nbw > (neqp-n)) lim = (neqp-n); else lim = nbw;

for(l=2;l<=lim;l++) {

k = n+l-1;

c[n] -= (a[n][l] * c[k]);

}

}

}

/*******************************/

void print_matrix(double **a,int m,int n)

{

int i,j;

for(i=1;i<=m;i++)   {

for(j=1;j<=n;j++)   {

printf("%12.4f",a[i][j]);

}

printf("\n");

}

printf("\n");

}

/*****************************/

void print_vector(double *v,int m)

{

int i;

for(i=1;i<=m;i++)   {

printf("%12.4f",v[i]);

}

printf("\n");

}

void matrix_multiply(double **a,double **b,double **c,int l,int m,int n)

{

int i,j,k;

double temp;

for(i=1;i<=l;i++)

{

for(j=1;j<=n;j++)

{

temp=0.0;

for(k=1;k<=m;k++)

{

temp += a[i-1][k-1]*b[k-1][j-1];

}

c[i-1][j-1]=temp;

}

}

}

/*******************************/

void matrix_transpose(double **a,double **b,int m,int n)

{

int i,j;

for(i=1;i<=m;i++)   {

for(j=1;j<=n;j++)   {

a[i-1][j-1]=b[j-1][i-1];

}

}

}

/*******************************/

void matrix_add(double **a,double **b,int m,int n)

{

int i,j;

for(i=1;i<=m;i++)   {

for(j=1;j<=n;j++)   {

a[i][j] += b[i][j];

}

}

}

/*******************************/

void matrix_copy( double **a, double **b, int m, int n)

{

int i,j;

for(i=1;i<=m;i++)   {

for(j=1;j<=n;j++)   {

a[i][j] = b[i][j];

}

}

}

/*******************************/

void matrix_scalar_multiply(double ** a, double **c, int m,int n)

{

int i,j;

for(i=1;i<=m;i++)

for(j=1;j<=n;j++)

a[i][j] = c[0][0];						//A Changer

}

/*******************************/

void matrix_vector_multiply(double **a,double *v,int k,int l,double *y)

{

int i,j;

double temp;

for(i=1;i<=k;i++)   {

temp=0.0;

for(j=1;j<=l;j++)   {

temp += a[i][j]*v[j];

}

y[j] = temp;

}

}

/*******************************/

void matrix_null(double **a,int m,int n)

{

int i,j;

for(i=1;i<=m;i++)   {

for(j=1;j<=n;j++)   {

a[i][j] = 0.0;

}

}

}

/*******************************/

void fmatrix_null(float **a,int k,int l,int m,int n)

{

int i,j;

for(i=k;i<=m;i++)   {

for(j=l;j<=n;j++)   {

a[i][j] = (float)0.0;

}

}

}

/*******************************/

void imatrix_null(int **a,int k,int l,int m,int n)

{

int i,j;

for(i=k;i<=m;i++)   {

for(j=l;j<=n;j++)   {

a[i][j] = 0;

}

}

}

/*******************************/

void dvector_copy(double *v,double *w,int n)

{

int i;

for(i=1;i<=n;i++)   {

v[i] = w[i];

}

}

/*******************************/

void vector_null(double *v,int n)

{

int i;

for(i=1;i<=n;i++)   {

v[i] = 0.0;

}

}

/*******************************/

void ivector_null(double *v,int n)

{

int i;

for(i=1;i<=n;i++)   {

v[i] = 0;

}

}

/*********************************/

/* convert double to nearest int */

/*********************************/

int d2i(double x)

{

double intpart;

double fraction;

fraction = fabs(modf(x,&intpart));

if( x >= 0.0)   {

if(fraction>0.5) return((int)(intpart+1));

return((int)(intpart));

}

else    {

if(fraction>0.5) return((int)(intpart-1));

return((int)(intpart));

}

}

## Appendix 7 : Isoparametric

This document was made with MathCAD. It allows to understand how work analytically the isoparametric method with an element (PLANE 42)

# References

M. L. Bittencourt, Using C++ templates to implement finite element classes, Engineering Computations, Vol.17 No7, 2000, pp. 775-778, May 200

http://www.santafe.edu/projects/swarm/swarmdoc/swarmdoc.html.

http://www.pa.msu.edu/courses/1996spring/PHY405.

B.W.R. Forde, R.O. Foshi, S.F. Stiemer, Object-oriented finite element analysis, Computers and structures, Vol 434, N°3, pp.355-374,1990

K.H.Ha, C language for finite element programming, Computers and structures, Vol 37, N°5, pp.873-880,1990

A. C. Staugaard, Structured and object-oriented techniques, 2nd Edition, 1996

I. Masters, J.T. Cross, R.W. Lewis, A brief review of object-oriented finite element methods, www.swansea.ac.uk.

D. Nash – CAED course – University of Strathclyde – 2002.

R.D. Cook, D.S. Malkus, M.E. Plesha, Concepts and applications of finite element analysis, 3rd edition, 1989

http://www.psi.toronto.edu/matrix/decomp.html.

W.H.Press, B.P.Flannery, S.A.Teukolsky, W.T.Vetterling, "Numerical Recipes in C, The Art of Scientific Computing," Cambridge University Press, 1988

L.E. Garey, A parallel numerical algorithm for near symmetric and banded system, Applied Mathematics and computation 119, pp 99-108, 2001.

www.algor.com.

www.ansys.com.

http://www.epm.ornl.gov/pvm/pvm_home.html.

http://junior.apk.net/~yaozheng/.

FE stands for finite element

A compiler is program which transforms a code readable by the developer to a code understandable for the computer.

TeraFLOP means 1012 floating points operations per second

Encapsulation is an advantage provided with the use of object-oriented program. It is explained later in the paper in more detail.

Libraries are files that we add to a program in order to include new functions…

OS : Operating system as UNIX, Microsoft Windows, etc …

address : each variable in memory has an address. The computer uses addresses automatically. The processor language does not understand the name of the variable and replace it by its address !

Low-level development consist with language very close to the process (with a few OS or functions already program). The opposite is a high level development.

a degree of freedom is a displacement allowed for a point of a body

A bijective function from a set D to a set is a rule that assigns a single element of R to each element in D.

header is a file where are define functions, classes, etc …